# K701 Failure Causal Learning

# Incident description:

On 22nd August 2017, K701B seized. K701A was supposed to switchover and run. However, the mechanical seal was found damaged. SM1 was then required to shut down as there is no vacuum pump for the SM1 section. When K701A was undergoing mechanical seal repair work, the shaft was found seized.

Upon dismantling of both pumps, the impellers of both pumps were found to have cracked.

# Problem Statement:

Expected:

K701B was supposed to run continuously and not seize. The standby pump was also supposed to switchover without any failure.

Actual:

K701B seized and stopped suddenly. Standby pump was unable to take over due to mechanical seal leak. Standby pump was also found seized when doing seal repair work.

Potential Impact:

This resulted in an unplanned down time incident for both SMPO1 and SMPO2. Total of 3.479kT losses (SMPO1: 2.04kT; SMPO2: 1.439kT)

# Sequence of Events:

| Before 2010 December | A: K701S B: K701B Spare: K701A |
| --- | --- |
| December 2010 | K701B mechanical seal leak A: K701S B: K701A Spare: K701B |
| April 2014 | K701B mechanical seal leak A: K701S B: K701B Spare: K701A |
| August 2016 | S1 TA2016 Start up Both K701A and K701B mechanical seal leak A: K701A B: K701B (mechanical seal still leaking) Spare: K701S |
| January 2017 | A: K701A B: K701S Spare: K701B |
| 19th June 2017 | K701S at B location tripped due to PDZ |
| 21st June 2017 | K701A at A location was found to be leaking (mechanical seal leak) K701S at B location was found to have seized when Ops tried to restart. |
| 22nd August 2017 | K701B at B location tripped due to PDZ. K701A restarted back and ran with leaking seal to shutdown SMPO1. |

# Cause Analysis:

Refer to the attached cause and effect diagram.

K701S

- Why did K701S trip on PDZ?

K701S failed to develop suction as the impeller vane broke. The suction loss triggered PDZ and hence tripped K701S.

- Why did K701S impeller vane break?

K701S impeller vane was found to have cracks and void. The hole act as a place to concentrate the stress. Eventually, the cracks propagated to the surface and cause the impeller vane to break.

K701B

- Why did K701B trip on PDZ?

K701B failed to develop suction as the impeller vane broke. The suction loss triggered PDZ and hence tripped K701B.

- Why did K701B failed to develop suction?

K701B failed to develop suction because the second stage impeller vane broke.

- Why did the second stage impeller vane broke?

The impeller vane is believed to be subjected to cyclic stress during the pump operation. Over the years, the stresses cause the vane to eventually fail.

# Observations, Insights, Conclusions:

| #### Observation |  |
| --- | --- |
| Insight |  |
| Conclusion |  |

# Recommendations:

(ST – Short Term / LT – Long Term)

| 1. (ST) Reversed engineer K701S second stage impeller |  |
| --- | --- |
| #### What cause should be addressed? | Impeller at K701S failed |
| Why? What would be the benefit? | K701S can be run. |
| How? What would be the solution? | NA |
| Action plan | Ivan Koh Reversed Engineered K701S second stage impeller (Completed) |

| 2. (ST) Repair the impeller for K701B |  |
| --- | --- |
| #### What cause should be addressed? | K701B impeller failed |
| Why? What would be the benefit? | K701B can be run |
| How? What would be the solution? | NA |
| Action plan | Ivan Koh Repair the impeller for K701B second stage. (Completed) |

| 3. (ST) Purchase a new set of impeller for K701B/K701S |  |
| --- | --- |
| #### What cause should be addressed? | NA |
| Why? What would be the benefit? | NA |
| How? What would be the solution? | NA |
| Action plan | Ivan Koh Purchase a new set of impeller for K701B/K701S. |

| 4. (ST) Purchase a new set of K701A |  |
| --- | --- |
| #### What cause should be addressed? | NA |
| Why? What would be the benefit? | NA |
| How? What would be the solution? | NA |
| Action plan | Ivan Koh Purchase a new set of K701A |

| 5. (ST) Perform radiography inspection on incoming impeller |  |
| --- | --- |
| #### What cause should be addressed? | NA |
| Why? What would be the benefit? | NA |
| How? What would be the solution? | NA |
| Action plan | Ivan Koh Perform radiography. (Completed) |

| 5. (LT) Perform PT on impeller vane when pump is removed for servicing. |  |
| --- | --- |
| #### What cause should be addressed? | This will detect any cracks on the surface which could act as stress concentration area, |
| Why? What would be the benefit? | Earlier detection of cracks will allow replacement to take place. |
| How? What would be the solution? | To include PT check into the checklist as part of the standard task when liquid ring pump is removed for mechanical seal change. |
| Action plan | Ruey Chyi Update Liquid Ring Pump repair checklist. (Completed) |

| - (LT) To perform internal inspection (SS) V704 in upcoming TA/opportunity, wall condition; weir plate condition etc. |  |
| --- | --- |
| #### What cause should be addressed? | This is to ensure the seal liquid level is always right at the vessel as well as at the liquid ring pump |
| Why? What would be the benefit? | Right liquid level will reduce the amount of stress the pump undergoes when it is running/ |
| How? What would be the solution? | Check on the weir plate condition and replace the equalizing valve. |
| Action plan | Ruey Chyi Raise GSAP for next TA2021 execution. (Completed) |

| - (LT) To convert LG (07LG011) to LT (similar type to 07LRCA009). |  |
| --- | --- |
| #### What cause should be addressed? | Ensure seal liquid level at the standby pump is correct from the DCS as the standby pump can start automatically. |
| Why? What would be the benefit? | Right liquid level will reduce the amount of stress the pump undergoes when it is running. |
| How? What would be the solution? | Convert LG to level transmitter. |
| Action plan |  |

| - (LT) To reroute seal liquid overflow line, direct from pump to discharge vessel. |  |
| --- | --- |
| #### What cause should be addressed? | Ensure seal liquid level at the standby pump is correct from the DCS as the standby pump can start automatically. |
| Why? What would be the benefit? | Right liquid level will reduce the amount of stress the pump undergoes when it is running. |
| How? What would be the solution? |  |
| Action plan |  |

| - (LT) To have individual PCV for mechanical seal plan 54 |  |
| --- | --- |
| #### What cause should be addressed? | Sudden change in seal pressure. |
| Why? What would be the benefit? | Allow individual control on the seal pressure. |
| How? What would be the solution? | Individual PCV for two liquid ring pumps. |
| Action plan |  |

# Timeline

# Cause and Effect Diagram:

The cause and effect diagram shows the cause and effect relationships to explain “why” the incident happened.

Attachments

- Matcor Failure Analysis Report

- PEFs for K701A/B system (with markup for seal liquid overflow line)

- K701S service report

- K701B Service report

- PI trend